|  | University of Hertfordshire - Individual Project Risk Assessment Form |  |
| --- | --- | --- |

| Project Title: Autonomous Underwater Robot (AUV) for Submerged Environment Scanning and Video Recording | Faculty: SET(RAI) | Batch No: 9 | Full Time / Part Time Full Time |
| --- | --- | --- | --- |
| Student Name: Kaung Linn Aung | Student No: 24123115 |  | Contact No: 9084minm@psba.edu.sg |

| SECTION B: RISK ASSESSMENT |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. HAZARD IDENTIFICATION |  |  |  | 2. RISK EVALUATION |  |  |  | 3. RISK CONTROL |  |  |  |
| No. | Work Activity | Hazard | Possible Injury /Ill-Health | Existing Risk Controls | Severity | Likelihood | RPN | Additional Controls | Severity | Likelihood | New RPN |
| 1 | Handling and soldering low-voltage electronics (Raspberry Pi, MCU, ESCs, wiring) inside the equipment sled | Electric shock; burns from soldering iron | Minor burns; electric shock | All electronics run on low-voltage DC (5-12V) only; soldering carried out in ventilated lab area; insulated tools used | 2 | 2 | 4 | Use fused/protected supply lines; wear safety glasses when soldering; unplug/rest soldering iron in stand when not in use | 2 | 2 | 2 |
| 2 | Use and charging of Lithium-Polymer (LiPo) battery sealed inside an unvented PVC pressure hull | Fire, thermal runaway, toxic/flammable gas release, explosion | Burns; smoke inhalation; chemical burns | Battery only charged outside the sealed hull using a dedicated LiPo balance charger in a fire-resistant charging bag; protected/fused battery pack used | 4 | 2 | 8 | Fit battery voltage/thermal monitoring circuit with automatic cut-off and physical fuse link; never charge battery while sealed inside the hull; keep a fire extinguisher/sand bucket nearby during all power-on testing | 4 | 2 | 4 |
| 3 | Testing the sealed AUV in a water test-tank | Electric shock; water ingress causing short circuit | Electric shock; electrical burns; equipment damage | Hull fully sealed and leak-tested before immersion; all onboard electronics powered only by the internal low-voltage battery (no mains power at tank side); internal moisture sensor fitted | 3 | 2 | 6 | Perform a dry pressure/leak test in an empty tank before first immersion; use RCD-protected supply for any tank pumps/lighting; second person (buddy) present during all water testing | 3 | 2 | 3 |
| 4 | Fabrication of the PVC pressure hull and mounting hardware using hand/power tools (cutting, drilling, sanding) | Cuts, abrasions, flying debris, dust inhalation | Cuts; eye injury; respiratory irritation | Work carried out in the university Mechanical/Electronics Lab under technician supervision; tools maintained and PPE provided by the lab | 3 | 2 | 6 | Complete lab induction/tool training before use; wear safety glasses and gloves at all times; use dust extraction/ventilation when sanding | 3 | 2 | 3 |
| 5 | 3D printing of PETG structural components (equipment sled, end-caps) | Burns from hot nozzle/heated bed; fume inhalation | Minor burns; mild respiratory irritation | Printer operated in a ventilated lab area under the lab's standard 3D-printing procedure | 2 | 2 | 4 | Do not touch nozzle/bed during or immediately after printing; run printer in a well-ventilated space; follow lab 3D-printing SOP at all times | 2 | 2 | 2 |
| 6 | Powering brushless motors/thrusters and propellers during bench and tank tests | Entanglement; cuts from rotating propellers | Cuts; pinching injuries | Motors only powered during controlled, supervised tests; propeller guard fitted where feasible | 3 | 2 | 6 | Power thrusters only when hull is submerged or clear of hands; accessible emergency stop/kill switch during testing; verbal warning given before powering motors | 3 | 2 | 3 |
| 7 | Use of epoxy potting compound / marine sealant for waterproofing the hull | Skin/eye irritation; inhalation of fumes | Skin irritation/dermatitis; eye irritation | Used in a ventilated area; Safety Data Sheet (SDS) reviewed before use | 2 | 2 | 4 | Wear nitrile gloves and safety glasses; work in a ventilated bench/fume area; wash hands thoroughly after handling | 2 | 2 | 2 |

| Student's Name & Signature: | Kaung Linn Aung | Sup's Name & Signature: | Wong Kim Siong | Lab Eng’s Name & Signature |  |
| --- | --- | --- | --- | --- | --- |
| Date: | 31 July 2026 | Date: |  |  |  |